[image: image2.jpg]Technologies, Ing



[image: image2.jpg]









[image: image3]

Broadband Linear RF Amplifier           3007
[image: image3]
· Frequency Response: 400-1000MHz
· Linear Power: 50 watt
· Saturated Power: 70 watts
· Gain: 48 dB
Description:
  The NP-2511 is a class A 20 watt CW, rack mountable amplifier system designed to operate over the frequency range of 400 MHz to 1000 MHz with a gain of 48db.  The NP-2511 operates from 95 to 255vac 47/63Hz, with RF input drive levels up to +2dBm.  NP-2511 is a self contained unit consisting of the RF power amplifier, AC to DC power supply, back panel shut down “BNC” connector and cooling fans.

	ELECTRICAL SPECIFICATION: Temp.=25°C, 50Ω System

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	400
	
	1000
	MHz

	Power Output Saturated
	Psat
	
	70
	
	Watt

	Power Output P-1dB
	P-1dB
	
	50
	
	Watt

	Gain
	G
	45
	48
	
	dB


	Small Signal Gain Flatness
	ΔG
	
	±0.5
	±1
	dB

	Input VSWR
	S11
	
	1.45:1
	1.55:1
	-

	Harmonics @ 50Watts Output
	H
	
	-40
	-28
	dBc

	Inter-modulation Point

2 Tones, 10W per tone @  959 & 960MHz
	IP3
	
	+55
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vac
	95
	
	255
	Volt

	Operating Current @ 100-120Vac
	Amps
	
	4.4
	
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical: 1ms OFF, 10ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	19 x 3.5  x 18.125
	Max
	Inch

	RF Connectors IN/OUT
	N
	-
	-

	DC Connectors
	N/A
	-
	-

	Cooling
	Heat-sink & Fans
	-
	-

	Weight
	20
	Typ
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 70°C with self reset.
	Typ

	Input Overdrive
	Fold-back overdrive protection to 20 dBm.
	Max

	Load VSWR
	4.0: up to 50 Watts
	Max

	Reverse Polarity Protection
	N/A
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+50°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
	[image: image4.jpg]dB

l:Reflection Log Mag 5.0 dBf Ref 0. 00 di
P2 Transmission Log Mag 1.0 dBf Ref 45.00 dB
Jul 21, Z20ps IBI:OZ Mgasl:Fkrl 400, 000 MHz
—15. 854dB
52
Measz2:Mkr2 1000, 000 MHz
- Level = 48. 002dB
+1.00| dBm
50
493
ff_xwx‘_""\ P _\_/_//“ =
>
f’A;J
47|
4F| \
45 — \
1 = \ \
' / / é\

Start 350.000 MHz

£ Stop 1 050,000 MHz






	Small Signal Frequency Response. 

	[image: image5.jpg]dB

1:Reflection Log Mag 5.0 dB/ Ref 0. 00 dB
P2 Transmission Log Mag 1.0 dB/ Ref 48.00 dB
Jul 21, 20p5 18: 00 Mgasl:Mkr1 400, 000 MHz
2 —13. 04%dB
Mgasz2:Mkrz 1000, 000 MHz
- Level = 49, 244dB
+22.00 dBm
50 e ] [ z
] N
4
, \
47 \
45 \
45 \
1
/N\‘\
44

/

7

Start 350.000 MHz

£ Stop 1 050.000 MHz






	Frequency Response Curve @ 50 Watt Output.


Response Curve
	[image: image1.jpg]dB

1:Reflection Log Mag 5.0 dB/ Ref 0,00 dB
P2 Transmission Log Mag 1.0 dBf Ref 48.00 dB
Jul 21, 2005 1@-:03 Mgasi:Hkrl 400, 000 MHz
- -18. 634dB
Measz:Mkrz 1000, 009 MHz
- Level = 46. 430dB
2.00 dBm
50
49
Ly
47 e E— W
46| \
45 \Y = \ \
T/ / A

Start 350,000 MHz

£ Stop 1 050.000 MHz





	Frequency Response Curve @ 70 Watts Output.


	[image: image6.jpg]o MKR a 1.995 HMH=z
REF -13.9 dBm ATTEH 18 dB -31.11 dB
PEAK : : y 9
LOB : : : '

Los | b 9
dB/

........................................................

................................................................................................

MARKER a
1.995 MH=z

"-31.11 dB

WA SB
Sc FC
CORR

CENTER 959.588 MHz SPAN 6.880 MH=z
#RES BW 18 kH=z VBW 18 kH=z SHP 180 msec





	Two Tones 10 Watts Avg. Per Tone @ 959 & 960MHz

IP3 = +55dBm




Outline Drawing
[image: image7.jpg]RF INPUT

RF POWER AMPLIFIER

RF OUTPUT














2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image8.jpg]


