[image: image2.jpg]Technologies, Ing



[image: image2.jpg]









[image: image3]

[image: image3]Linear RF Amplifier                                       
· Frequency Response: 100KHz - 200MHz
· Linear Power: 50 watts 
· Saturated Power: 80 watts 
· Gain: 53 dB
Description:
 Designed for linear application in the 100 KHz to 200 MHz range. This amplifier utilizes RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. 
Suggested applications: CW, multi-carrier, AM & FM modulation.














   Updated: 0808
	ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	0.100
	
	200
	MHz

	Power Output Saturated 
	Psat
	
	80
	
	Watt

	Power Output P-1dB 
	P-1dB
	
	50
	
	Watt

	Gain
	G
	50
	53
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	±1
	±1.5
	dB

	Input VSWR
	S11
	
	1.45:1
	2.0:1
	-

	Harmonics @ 50watts 2nd/3rd 
	H
	
	-44/-24
	
	dBc

	Inter-modulation Point

2 Tones, 1W per tone @  50 & 51MHz
	IP3
	
	55
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vac
	100
	115
	240
	Volt

	Operating Current
	Amps
	
	
	11
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical TBD OFF, TBD ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	19” x 17”deep, 3U Rack
	Max
	Inch

	RF Connectors IN/OUT
	N in, N out
	-
	-

	Control Connector
	BNC/15 pin D-sub 
	-
	-

	Cooling
	Fan forced heat-sink
	-
	-

	Weight
	14
	Typ
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 70°C with self reset.
	Typ

	Input Overdrive
	+0dBm Max
	Max

	Load VSWR
	4.0:1 up to 50 watts
	Max

	Reverse Polarity Protection
	None
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+50°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
[image: image1.jpg]Trl 511 Log Mag
FIEE 51 Log Mag
S5&.00

B/ Ref 0.000dE [RS]
.000dBy Ref 53.00d4B [RT D&M]

E1.00  fevmmne

&0.00

49.00

1 300.00000 kHz G3.724
z 10.000000 WHz 52.572
R 3 ©D.0000CD MHz 52.845
| 5700 |eq-100,00000 Rz 521551
5 1EQ.00000 MHz 51.988
»& 200.00000 MHz 52.216
56.00 : ;

oo
H0 03 00 0 07

48.00

Start 200 kHz

IFEI 70 kHe




Small Signal Frequency Response Curve /
Frequency Response Curve @ 50 Watt Outp









2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image4.jpg]RF INPUT

MODEL

# NP-2

519

RF POWER AMPLIFIER

100KHz - 200MHz

RF OUTPUT

50 Watts,

53dB GAIN



