[image: image1.jpg]Technologies, Ing



[image: image1.jpg]









[image: image2]

[image: image2]Class “A” Linear RF Amplifier              
· [image: image3.png]B

El:Reflection Log Mag 5.0 dB/ Ref 0.00 dB
»2: Transmission Log Mag 1.0 dB/ Ref 44.00 dB
e vz 500.000 MHz
u 2. 03308
rZ 500.000 MHz
Level = 44. 15808
47
22.00 dBm
4
4
L 2
4
4
41, —
e
p

Start 0.300 MHz

Stop 550.000 MHz




Frequency Response: 20-500 MHz
· Linear Power: 20 watts class “A”
· Saturated Power: 50 watts
· Gain: 43 dB
Description:
 Designed for linear application in the 20 to  500 MHz range. This amplifier utilizes
 class “A” RF Power MOSFET devices that provide high gain, wide dynamic range and 
	an excellent 3rd order intercept point. Suggested applications: multi-carrier, pulse, AM & FM modulation.
ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System                                            0513

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	20
	
	500
	MHz

	Power Output Saturated
	Psat
	50
	
	
	Watt

	Power Output P-1dB
	P-1dB
	25
	30
	
	Watt

	Gain
	G
	41
	44
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	±0.8
	±1.3
	dB

	Input VSWR
	S11
	
	1.5:1
	2.0:1
	-

	Harmonics @ 20Watts: 20-100 / 150-500 (MHz)
	H
	
	
	-26 / -28
	dBc

	Inter-modulation Point

2 Tones, 1W per tone @ 449 & 450 MHz
	IP3
	
	+54
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current @ 20 Watts / 50 Watts
	Amps
	
	6.8
	8.5
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical: 1ms OFF, 10ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions: Module / Module + Heatsink
	5.30x2.50x1.05 / 9.75x7.30x6.5
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through.
	-
	-

	Cooling: “Optional”
	4” Fan and Heat-sink.
	-
	-

	Weight:  Module / Module + Heatsink
	1 / 8.5
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 80°C with self reset.
	Typ

	Input Overdrive
	Fold-back overdrive protection to 20 dBm.
	Max

	Load VSWR
	Infinite up to 20 watts.
	Max

	Reverse Polarity Protection
	Included
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	70°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
[image: image4.jpg]


[image: image5.jpg]THRU 4 PLACES.

Ground /ShutDFF 5025 +28vdc. / Graund
\ o o L \ o
A |
2,500 @ Eiee0:
‘ L

Ofren By G0

0486 | o M, o

1.000
L]

LI
RF INPUT 1‘0.50 o

5.300
6.170

B
Ty RF OUTPUT



Small Signal Frequency Response Curve 

Frequency Response Curve @ 20 Watts Output
[image: image6.png]El:Reflection Log Mag 5.0 dB/ Ref 0.00 dB
»2: Transmission Log Mag 1.0 d/ Ref 44.00 dB
a8 tdas1:MErz  00. 00 MHz
u -£0.95{dB
Melas2: Mkrz  500.000 MHz
4 Level 43. 664dB
.00 dBm
4
¢ 7~ [t
b 2
4
4
41
L
2
p

Start 0.300 MHz

Stop 550.000 MHz




[image: image7.png]E1:Reflection Log Mag 5.0 dB/ Ref 0.00 dB

»2: Transmission Log Mag 1.0 dB/ Ref 44.00 dB
tdas1: Mz 00. 00 MHz
-20. 57308
Melas2: Mkrz  500.000 MHz
Level 42. 973dB
4.00 dBm
—

Start 0.300 MHz Stop 550.000 MHz



Response Curves
Frequency Response Curve @ 50 Watts Output

Two Tones 5 Watts Avg.  Per Tone @ 449 & 450MHz

IP3 = +54dBm

Outline Drawing: Module,
[image: image8.jpg]o
REF -
PEAK

LOG
i@
dB/

WA SB
SC FC
CORR

CENTE

1.8 dBm ATTEN 18 dB

MKR & 2.818 MH=z
-34 .96 dB

MARKER &
2.818 MH=z

'-34.96 dB

...........................................................................................

R 449.588 MH=z
RES BW 38 kHz UBW 328 kH=z

SPAH 6.886 HWMH=z
SWUP 268.8 msec




Outline Drawing: Module + Heatsink,

[image: image9.png]100

9,754
2,500

|~u sLom 6 PLes.

N —

| 135 RAD.TYP,

WP Toshnalogles, Ino. -
7300 4.800 PR EN 6000
drms
1m0
2 N
T
| b

HEATSINK
ANPLIFIER MODULE
DETAIL







Heatsink Optional








2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

All specifications subject to change without notice

