[image: image1.jpg]Technologies, Ing



[image: image1.jpg]









[image: image2]

[image: image2] Class “A” Linear RF Amplifier          
[image: image3.jpg]



· Frequency Response: 500-1000 MHz
· Linear Power: 20 watt
· Saturated Power: 50 watts
· Gain: 49 dB
Description:
 Designed for linear application in the 500 to 1000 MHz range. This amplifier utilizes
 class A RF Power MOSFET devices that provide high gain, wide dynamic range and 
an excellent 3rd order intercept. Suggested applications: multi-carrier, pulse, AM & FM 
	modulation.
ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System                                             0513

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	500
	
	1000
	MHz

	Power Output Saturated
	Psat
	
	50
	
	Watt

	Power Output P-1dB
	P-1dB
	20
	
	
	Watt

	Gain
	G
	45
	49
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	
	±1
	dB

	Input VSWR
	S11
	
	1.45:1
	1.7:1
	-

	Harmonics @ 20 Watts 500-600/650-1000 (MHz)
	H
	
	-40
	-32 / -35
	dBc

	Inter-modulation Point

2 Tones, 1W per tone @  950 &  951MHz
	IP3
	
	+54
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current @ 20 Watts / 50 Watts
	Amps
	
	6.6 / 8.2
	
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical: 1ms OFF, 10ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions: Module / Module + Heatsink
	6.00x2.95x1.04 / 9.75x7.30x6.50
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through
	-
	-

	Cooling: “Optional”
	Heat-sink and Fan
	-
	-

	Weight:  Module / Module + Heatsink
	1 / 8.75
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 80°C with self reset.
	Typ

	Input Overdrive
	Fold-back overdrive protection to 20 dBm
	Max

	Load VSWR
	Infinite up to 20 watts
	Max

	Reverse Polarity Protection
	Included
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+70°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
[image: image4.png]=l:Retlection Log Mag

bl Transmission Log Mag

CINC L
IIHIIEH

-
-
A
-
o
-

SEEEERNEOE
UNEENE

N

Start 450, 000 MHz

dbB! Ret

L, 0Ol ds

dgs/ Ret 47,00 dB

St o

1

D50, Do)

HMH=z




	

	Small Signal Frequency Response Curve  

	[image: image5.png]dbB! Ret L, 0Ol ds
dgs/ Ret 47,00 dB

=l:Retlection Log Mag .
bl Transmission Log Mag 1

Start 450, 000 MHz Stﬂp D50, DDD HMH=z





	Frequency Response Curve @ 20 Watt Output


Response Curve
	[image: image6.png]wliketlection Leg Mag 5.0 db/ Ret  0.0U0 db

»2:Transnission Log Mag 1.0 dB/ Ref 47.00 dB
HdasT:Mkr1 500, 60§ MHz
5. 11308
Mdas2:Mkr2 1000. GO0 MHz
Level = 45. 941 dB
1.00 dBm

Start 450. 000 MHz Stop 1 050.000 MHz





	Frequency Response Curve @ 50 Watts Output 



	[image: image7.jpg]-
REF -
PEAK

LOG
18
dB/

WA SB
sSC FC
CORR

CENTE

MKR & 1.988 HMHz

15.8 dBm ATTEN 18 dB -38.91 dB
T SO SOV O UPURTE S « U PN SO PR UL ST SO UUTOI PR
RIS | HNNIIHIE | PTIN SOI
-1.988 NHZ ------------------------------------------------
-38.91 dB
*

R_949.508 MHz SPAN 6.888 MHz
RES BW 38 kHz VBMW 38 kHz SWP 28.8 mseo





	Two Tones 5 Watts Avg. Per Tone @ 949 & 950MHz 
IP3 = +56dBm




Outline Drawing: Module,
[image: image8.jpg]0135
— #0166 THRU 4 PLACES,
5,730
Ground /Smmw [’ /+88vdc Ground
&)
\ n) ‘ ] sHuT-orF +z/s/v§*ﬂ A ©/
NP Technolegies, Inc. ‘
f@ ohe Of 2t e ) 2680 ) E@T 2,95
1.48 \ \ 1.48
| %o B |
- 0.135 6.000 0.310
0,310
RF INPUT RF OUTPUT
7 3 L AMPLIFIER
1‘0*42 0 0 0780 MODULE

f

DETAIL




Outline Drawing: Module + Heatsink
[image: image9.png]9.754-

8.500——————|

|~u sLom 6 PLes.

| 135 RAD.TYP,

|

S adoo lof W Teomotgion e, om0
N Lo
0925
150
b ﬂ ﬂ 0650
T
050
7.000-
e b
of
6242

HEATSINK
ANPLIFIER MODULE








Heatsink


Optional








2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

All specification subject to change without notice

