[image: image1.jpg]Technologies, Ing



[image: image1.jpg]









[image: image2]

 AM Broadcast Band Amplifier
[image: image2]
· Frequency Response: 0.53 -1.7 MHz
· Linear Power: 200 watt
· Saturated Power: 250 watts
· Gain: 65 dB
Description:
 Designed for high power application in the 530 KHz to 1.7 MHz AM broadcast band range. Suggested applications: multi-carrier, pulse, AM & FM modulation.
	ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System                                            0513

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	0.53
	
	1.7
	MHz

	Power Output Saturated
	Psat
	
	250
	
	Watt

	Power Output P-1dB
	P-1dB
	180
	200
	
	Watt

	Gain
	G
	63
	65
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	
	±0.5
	dB

	Input VSWR
	S11
	
	1.3:1
	1.4:1
	-

	Harmonics @ 200 Watts 2nd/3rd
	H
	
	-35 / -25
	-28 / -20
	dBc

	Inter-modulation Point

2 Tones, 5W per tone @ 1 & 1.1 MHz
	IP3
	
	+60
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current @ 200 Watts
	Amps
	
	16
	20
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical: 1ms OFF, 10ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	8 x 4.9 x 1.25
	Max
	Inch

	RF Connectors IN/OUT
	N
	-
	-

	DC Connectors
	Filtered feed-through
	-
	-

	Cooling
	Heat-sink not included
	-
	-

	Weight
	2.5
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 80°C with self reset.
	Typ

	Input Overdrive
	0dBm Max
	Max

	Load VSWR
	3.0:1 up to 200 Watts
	Max

	Reverse Polarity Protection
	Included
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+70°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
[image: image3.jpg]



	

	[image: image4.png]671

E1:Reflection Log Mag 5.0 dB/ Ref 0.00 dB C?

»2: Transmission Log Mag 1.0 dB/ Ref 65.00 d§ C?
dasl:Mkrz | 1,700 MHz
21 39508
Measz:Mkrz | 1.700 MHz
Level = 64. 65608
33.00 dBm
o] 2

it

Start 0.300 MHz Stop 2.000 MHz




Small Signal Frequency Response Curve 

	

	Frequency Response Curve @ 100 Watts Output


Response Curve
[image: image5.png]671

E1:Reflection Log Mag 5.0 dB/ Ref 0.00 dB C?

»2: Transmission Log Mag 1.0 dB/ Ref 65.00 d§ C?
dasl:Mlrz | 1,700 MHz
-23. 14908
Measz:Mkrz | 1.700 MHz
Level 65.031dB
15.00 dBm
et e — 2
A —
1] I e N

Start 0.300 MHz Stop 2.000 MHz




	

	Frequency Response Curve @ 150 Watts Output 

	[image: image6.png]671

E1:Reflection Log Mag 5.0 dB/ Ref 0.00 dB C?

»2: Transmission Log Mag 1.0 dB/ Ref 65.00 d§ C?
2003{06-11[ 16:59: 55 dasl:Mkrz | 1,700 MHz
21, BB
Measz:Mkrz | 1.700 MHz
Level 65. 018dB
13.00 dBm
2
—r—
T ]
2
<_//”§Y4' I —

Start 0.300 MHz Stop 2.000 MHz





	Frequency Response Curve @ 200 Watt Output


[image: image7.png]671

E1:Reflection Log Mag 5.0 dB/ Ref 0.00 dB C?

»2: Transmission Log Mag 1.0 dB/ Ref 65.00 d§ C?
dasl:Mlrz | 1,700 MHz
-z4.51{d8
Measz:Mkrz | 1.700 MHz
Level 63. 95808
11.00 dBm
2
e L | |

Start 0.300 MHz Stop 2.000 MHz




	

	Compression at P-1 db 

	[image: image8.png]dasl:Mkrz | 1,700 HHz
o -23.51{d8
Measz:Mkrz | 1.700 MHz
. Level 64. 248dB
11.50 dBm
67
[
[ g (A R——
]
e e ENUSUIS DR NS A v
N 5 S
6
3
61 =
=17 — |

E1:Reflection Log Mag
»2:Transmission &M Log Mag

5.0 dB/ Ref 0.00 dB C?
1.0 dB/ Ref E5.00 dB_ C?

Start 0.300 MHz

Stop 2.000 MHz



[image: image9.jpg]4
REF -
PEAK

WA SB
SC FC
CORR

CENTE
#

MKR & 160.5 kHz

7.8 dBm ATTEN 48 dB ~28.35 dB

MARKER a
188.5 kHz

'-38.,35 dB

SPAN £08.6 kHz
SHP 280 msec

R i.@5@8
RES BHW

NE

NE






	Two Tones 12.5 Watts Avg. Per Tone @ 1 & 1.1 MHz

IP3 = +60dBm


Outline Drawing
[image: image10.png]2400 5200

) S R
B
Status b
Shut Town Pt
i ° +28VIC "Bt
f o o o o o o o
\ |
i
i i i)














2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

