[image: image1.jpg]Technologies, Ing



[image: image1.jpg]









[image: image2]

[image: image2] Broadband Linear RF Amplifier
· Frequency Response: 10-1000 MHz
· Linear Power: 1 watt
· Saturated Power: 2 watts
· Gain: 11 dB
Description:
 Designed for linear application in the 10 to 1000 MHz range. This amplifier utilizes RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. Suggested applications: multi-carrier, pulse, AM & FM modulation.
	ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	10
	
	1000
	MHz

	Power Output Saturated
	Psat
	
	2
	
	Watt

	Power Output P-1dB
	P-1dB
	
	1
	
	Watt

	Gain
	G
	10
	11
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	
	±1.0
	dB

	Input VSWR
	S11
	
	1.3:1
	1.5:1
	-

	Harmonics @1 Watt
	H
	
	-28
	-20
	dBc

	Inter-modulation Point

2 Tones, 100 mW Per tone @ 900 & 901 MHz
	IP3
	
	+36
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current
	mAmp
	
	300
	450
	mAmp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Not Included
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	2.0 x 1.6 x .075
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through.
	-
	-

	Cooling
	Heat-sink not included
	-
	-

	Weight
	
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	None
	Typ

	Input Overdrive
	+24 dBm Max
	Max

	Load VSWR
	Infinite up to 1 Watt
	Max

	Reverse Polarity Protection
	None
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+70°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
	[image: image3.png]1B

wlihetlection Log Mag 5.0 do/ ket 0.0UU db
»2: Transmission Log Mag 1.0 dB/ Ref 11.00 dB
tdas1:Mrz 1000, 00 HHz
8. 11208
Melas2: Mkrz 1000. 000 MHz
Level = 10.410d8
20.00| dBm
i i S SR

o

Start 0.300 MHz
Start 0,300 MHz

Stop 1 000.000 MHz

Stop 1

100. 000 MHz






	Small Signal Frequency Response Curve 

	[image: image4.png]1B

wliRetlection

Log Mag

2.0 db/ Ret  0.00 db

»2: Transmission Log Mag 1.0 dB/ Ref 11.00 dB
tdas1:Mrz 1000. 00 HHz
23, 6808
Melas2: Mkr2 1000. 000 MHz
Level 11.238d8
.00 dBm
[ g

Start 0.300 MHz
Start 0,300 MHz

Stop 1 000.000 MHz
Stop 1 100.000 MHz






	Frequency Response Curve @ 0.5 Watt Output


Response Curve
	[image: image5.jpg]




	Frequency Response Curve @ 1 Watt Output

	[image: image6.jpg]19:28:514 MAR @83, 2064

e
REF -32.8 dBm ATTEN 10 dB

PEAK
LOG
ip
dB/

m=
OO
amm
A m

MKR & -1.885 MH=z
-32.7Y6 dB

MARKER a
-1.885 MH=z

'~-32.7E6 dB

CEMTER 9806.588 MH=z
#RES BW 1.8 kH=z

.88 MHz
VBW 1 kH=z P 18.@ sec





	Two Tones 100 mwatts Avg.  Per Tone @ 900 & 901MHz

IP3 = +36dBm




[image: image7.png]r1‘785*
k3 /@[1150

1.600

INPUT OUTPUT

3 f
m@ NS0T o 1‘3l25

o ® o

!

~—2.000—

0.750

f

i o
Pagm—l



Outline Drawing









2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image8.png]1B

wliRetlection

Log Mag

2.0

de/ Ret  0.00 db

»2: Transmission Log Mag 1.0 dB/ Ref 11.00 dB
tdas1:Mrz 1000, 00 HHz
8. 35308
Melas2: Mkr2 1000. 000 MHz
Level = 10. 50008
16.00| dBm
[ —

o

Start 0.300 MHz
Start 0,300 MHz

Stop 1 000.000 MHz

Stop 1

100. 000 MHz




