[image: image1.jpg]Technologies, Ing



[image: image1.jpg]









[image: image2]

 Broadband Linear RF Amplifier


   


[image: image2]
· Frequency Response: 10-1000 MHz
· Linear Power: 1 watt
· Saturated Power: 2 watts
· Gain: 23 dB
Description:
 Designed for linear application in the 10 to 1000 MHz range. This amplifier utilizes RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. Suggested applications: multi-carrier, pulse, AM & FM modulation.
	ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System                                            0513

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	10
	
	1000
	MHz

	Power Output Saturated
	Psat
	
	2
	
	Watt

	Power Output P-1dB
	P-1dB
	
	1
	
	Watt


	Gain
	G
	20
	23
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	
	±1.0
	dB

	Input VSWR
	S11
	
	1.3:1
	1.5:1
	-

	Harmonics @ 1 Watt 2nd/3rd
	H
	
	-30 / -36
	-28 / -30
	dBc

	Inter-modulation Point

2 Tones, 100mW per tone @ 900 & 901 MHz
	IP3
	+38
	+40
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current
	mAmps
	
	750
	850
	mAmp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Not included
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	3.5 x 1.6 x 0.75
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through
	-
	-

	Cooling
	Heat-sink not included
	-
	-

	Weight
	
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	None
	Typ

	Input Overdrive
	+13 dBm Max
	Max

	Load VSWR
	Infinite up to 1 Watt
	Max

	Reverse Polarity Protection
	None
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+70°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
	[image: image3.png]wliRetlection Log Mag 5.0 do/ ket 0U.0U db

»2: Transmission Log Mag 1.0 dB/ Ref 22.00 dB
tdas1:Mrz 1000. 00d HHz
-37. 74508
Melas2: Mkr2 1000. 000 MHz
Level 23. 22348
10.00 dBm
2

Start 0.300 HHz Stop 1 000.000 MHz
Start 0,300 MHz Stop 1 100.000 MHz





	Small Signal Frequency Response Curve 

	[image: image4.jpg]




	Frequency Response Curve @ 0.5 Watt Output


Response Curve
	[image: image5.png]wliRetlection Log Mag 5.0 do/ ket 0U.0U db

»2: Transmission Log Mag 1.0 dB/ Ref 22.00 dB
tdas1:Mrz 1000, 00 HHz
-36. 5778
Melas2: Mkr2 1000. 000 MHz
Level 22. 404dB
5.00 dBm
£

/|

Start 0.300 HHz Stop 1 000.000 MHz
Start 0,300 MHz Stop 1 100.000 MHz





	Frequency Response Curve @ 1 Watt Output 

	[image: image6.png]1B

wliRetlection Log Mag 5.0 do/ ket 0U.0U db

»2: Transmission Log Mag 1.0 dB/ Ref 22.00 dB
tdas1:Mrz 1000. 00 MHz

. -33. 6978
Melas2: Mkr2 1000. 000 MHz

2 Level 22. 476dB

8.00 dBm

2

Y

21

2

1

. e

Start 0.300 MHz Stop 1 000.000 MHz
Start 0,300 MHz Stop 1 100.000 MHz




Two Tones 100 mwatts Avg.  Per Tone @ 900 & 901MHz

IP3 = +38dBm




Outline Drawing
[image: image7.jpg]19:33:43 MAR B3, 2864
s MKR a -1.885 MH=z

REF —-22.8 dBm ATTEN 18 dB -36.96 dB

PEAK : : : : : 2 : : :

LOB : ; : : ; : ; : ;

1@ ...... T ——— AR RAREEECE R Q ....... i A AAEEELEE SARREEELL  BRE R A LS E R
T T e B e K S S

-1.885 MHz
-36.96 dB

WA 5B
S5C FC
CORR

CEMTER 988.588 MH=z SPAN &6.888 MH=z
#RES BW 1.8 kH=z VBW 1 kHz SHP 18.8 sec













2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image8.png]32205 —
3| 20150
] @ ] + =l
'if/
|:ﬂ:ﬂ:g}NPUT wrmeee OUTPUTQ:H:“] 1‘385
o @ @ o
3500
t
0.750 i m
|

4.370



