[image: image1.jpg]Technologies, Ing



[image: image1.jpg]









[image: image2]

[image: image2] Pulse RF Amplifier
· Frequency Response: 380-420 MHz
· Power: 10 watts PEP
· CW Power: 3 watts
· Gain: 38 dB
Description:
 Designed for driving an acousto-optic (Bragg) cell. This amplifier utilizes RF Power MOSFET devices that provide high gain, wide dynamic range and great pulse reproduction with minimal pulse ringing.
	ELECTRICAL SPECIFICATION @ VDD= +24VDC: Temp.=25°C, 50Ω System                                             0613

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	380
	
	420
	MHz

	Power Output CW
	PCW
	
	3
	
	Watt

	Power Output PEP (10 ns pulse 25% duty cycle)
	PPEP
	
	10
	
	Watt

	Gain
	G
	35
	38
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	
	±1.0
	dB

	Input VSWR 
	S11
	
	1.6:1
	2.0:1
	-

	Harmonics @ 3 Watts CW
	H
	
	-19
	-18
	dBc

	Inter-modulation Point

2 Tones, 0.5W per tone @ 380 & 381 MHz
	IP3
	
	50
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	23
	24
	28
	Volt

	Operating Current
	Amps
	
	1.5
	2.0
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Not Included
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	2.2 x 4 x 0.86
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through
	-
	-

	Cooling
	Heat-sink not included
	-
	-

	Weight
	1
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	None
	Typ

	Input Overdrive
	+5 dBm PEP Max
	Max

	Load VSWR
	Infinite up to 10 watts PEP
	Max

	Reverse Polarity Protection
	None
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+70°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
	[image: image3.png]E1:Reflection Log Mag 5.0 dB/ Ref 0.00 dB

»2: Transmission Log Mag 2.0 dB/ Ref 37.00 dB
tdas:MEr1 380, 000 MHz
12, B5%d8
Melas2:Mkr1  380.000 MHz
Level = 38. 66208
22.00 dBm

Start 10.000 MHz Stop 800.000 MHz





	Broadband Small Signal Frequency Response Curve 

	[image: image4.jpg]




	Small Signal Frequency Response Curve


Response Curve
	[image: image5.png]1B
41

E1:Reflection
»2:Transmission

Log Mag
Log Mag

5.0 dB/ Ref  0.00 dB
1.0 dB/ Ref 37.00 dB

tdas1:Mlrz 420,000 MHz
12.51{d8
Melas2: Mkrz 420000 MHz
Level = 38. 59608
22.00 dBm

Start 350.000 MHz

Stop 450.000 MHz





	10 Watt Frequency Response Curve 

	[image: image6.png]E1:Reflection Log Mag 5.0 dB/ Ref 0.00 dB

»2: Transmission Log Mag 1.0 dB/ Ref 37.00 dB
tdas1:Mlrz 420,000 HHz
H12.817d8
Melas2: Mkrz 420000 MHz
Level 37. 95308
2.00 dBm
% g

Start 350.000 MHz Stop 450.000 MHz





	Two Tones 0.5 Watts Avg. Per Tone @ 380 & 381 MHz
IP3 = +50dBm


Response Curve
	[image: image7.jpg]I g
REF
PEAK
LOG
1@
dB ¢

WA S
sSC F
COR

CENTER 388.588 MH=z

MKR & =998 kHz
-25.8 dBm ATTEN 18 dB -46.44 dB
....................................... B o ———-
'ﬁﬁﬁkﬁﬁ"g ...............................................................................
~-998 kH=z

"-45.44 dB

B
C
R

#RES BW 18 kH=z

UBW 18 kH=z

SPAN 5.888 MH=z
SHP 188 msec





	Source: Coherent Cavity Dumper 7200 

	[image: image8.jpg]




	NP-512 @ 10 watts PEP


Outline Drawing
[image: image9.jpg]. | — -













2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image10.png]" 3.750 T 60156

55 ® a1

/ ‘
NP Technologies, Inc.
MODEL # NP-512
1,950 ® @ 2,200
[ﬂ:ﬂ: INPUT OUTPUT]:H:H]
lo & GND +24 V() \
F T
~ 4,000 - 0.862

4,870

8



