[image: image1.jpg]Technologies, Ing



[image: image1.jpg]









[image: image2]

[image: image2]Linear RF Amplifier
· Frequency Response: 112-137 MHz
· Linear Power: 80 watt
· Saturated Power: 125 watts
· Gain: 55 dB
Description:
 Designed for linear application in the 112 to 137 MHz range. This amplifier utilizes RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. Suggested applications: multi-carrier, pulse, AM & FM modulation. (AM distortion < 2% @ 25 Watt Carrier, 90% Modulation.)
	ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System                                             0613

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	112
	
	137
	MHz

	Power Output Saturated
	Psat
	
	125
	
	Watt

	Power Output P-1dB
	P-1dB
	80
	100
	
	Watt

	Gain
	G
	50
	55
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	
	±0.3
	dB

	Input VSWR
	S11
	
	1.2:1
	1.3:1
	-

	Harmonics @ 80 Watts
	H
	
	-24
	-20
	dBc

	Inter-modulation Point

2 Tones, 12.52W per tone @ 118 & 119 MHz
	IP3
	
	+60
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current @ 80 Watts
	Amps
	
	5.4
	6.5
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical 25ms OFF, 450ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	5 x 3.35 x 1.40
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through
	-
	-

	Cooling
	Heat-sink integrated. Fan required
	-
	-

	Weight
	1.5
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 80°C with self reset.
	Typ

	Input Overdrive
	-3dBm Max
	Max

	Load VSWR
	4.0:1 up to 80 watts
	Max

	Reverse Polarity Protection
	None
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+70°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
	[image: image3.jpg]




	Small Signal Frequency Response Curve Broadband

	[image: image4.png]B

El:Reflection Log Mag 10.0 dB/ Ref 0.00 dB C?
»2: Transmission Log Mag 5.0 dB/ Ref 55.00 d8  C?
tdas:Mkrz 1137, 000 MHz
-30. 73308
Melasz:Mkrz 1137000 MHz
Level = 55. 634dB
26.00 dBm
2
&
]
2
/ Vi

Start 0.300 MHz

Stop 300.000 MHz





	Small Signal Frequency Response Curve


Response Curve
	[image: image5.png]571

E1:Reflection Log Mag 10.0 dB/ Ref 0.00 dB C?

»2: Transmission Log Mag 1.0 dB/ Ref 55.00 d8 C?
tdas:Mkrz 1137, 000 MHz
-31. 36308
Melasz:Mkrz 137,000 MHz
Level = 55. 434dB
26.00 dBm

Start 100.000 MHz Stop 150.000 MHz





	Frequency Response Curve @ 100 Watts 

	[image: image6.png]571

E1:Reflection Log Mag 10.0 dB/ Ref 0.00 dB C?

»2: Transmission Log Mag 1.0 dB/ Ref 55.00 d8 C?
tdas:Mkrz 1137, 000 MHz
-32. 004dB
Melas2:Mkrz 137,000 MHz
Level = 54. 96008
5.00| dBm
2
T R S—
— 2

Start 100.000 MHz Stop 150.000 MHz





	Two Tones 12.5 Watts Avg.  Per Tone @ 118 & 119MHz

IP3 = +60dBm



	


Outline Drawing
[image: image7.jpg]14:29:18 MAR 26, 2883

REF ~12.8 dBm ATTEN 18 dB

PEAK
LOG
18
dB/

WA SB
SC FC
CORR

MKR a -2.0088 MHz
-38.68 dB

MARKER a
-2.8B8 MHz

'-38.63 dB

.......................................................

CENTER 118.508 MH=z
#RES BW 18 kH=z

SPAN 8.8668 MH=z
VBW 18 kH=z SHUP 248 msec













2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image8.png]rosse

2350 2.200

2639

1400

5000

s0i5e x4
s
— F=3
= +| 3000 2350 =
= i E
E=d - ==
=]
— s



