[image: image1.jpg]Technologies, Ing



[image: image1.jpg]









[image: image2]

[image: image2]Linear RF Amplifier
· Frequency Response: 225-400 MHz
· Linear Power: 15 watt
· Saturated Power: 20 watts
· Gain: 15 dB
Description:
 Designed for linear application in the 225 to 400 MHz range. This amplifier utilizes class AB RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. Suggested applications: multi-carrier, pulse, AM & FM modulation.
	ELECTRICAL SPECIFICATION @ VDD= +24VDC: Temp.=25°C, 50Ω System                                             0613

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	225
	
	400
	MHz

	Power Output Saturated
	Psat
	
	20
	
	Watt

	Power Output P-1dB
	P-1dB
	13
	15
	
	Watt

	Gain
	G
	13
	15
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	
	±0.5
	dB

	Input VSWR
	S11
	
	1.4:1
	1.5:1
	-

	Harmonics @ 10 Watts: 2nd / 3rd
	H
	
	-30
	-20 / -23
	dBc

	Inter-modulation Point

2 Tones, 2W per tone @ 350 & 351MHz
	IP3
	+52
	+54
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	22
	24
	28
	Volt

	Operating Current @ 10Watts
	Amps
	
	1.6
	2
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Not Included
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	2.2 x 4 x 0.86
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through
	-
	-

	Cooling
	Heat-sink not included
	-
	-

	Weight
	1
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 80°C with self reset.
	Typ

	Input Overdrive
	+28 dBm Max
	Max

	Load VSWR
	6.0:1 up to 10 Watts
	Max

	Reverse Polarity Protection
	None
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+70°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
[image: image3.jpg]M9 Plag  RLAELS Rel U, BYER

»2: Transmission Log Mag 1.0 dB/ Ref 15.00 dB
2003f11-25] 17: 20: 56 tdas1:Mkrz 400 00 MHz
2. 41008
Melas2: Mkrz  400. 000 MHz
Level = 16. 66508
10.00| dBm
17 P g
&

Start 100.000 MHz Stop 500.000 MHz





	

	[image: image4.png]1,950

@

OUTPUT j:ﬂ:ﬂ]

GND +24 VO

TP

4,000 -

4,870

8

LB




Small Signal Frequency Response Curve 

	

	


Outline Drawing
[image: image5.jpg]












2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image6.png]VDS = 24vdc, IDQ = 1.04

Frequency | Pin Pout | Current Harmonics P-1d8
MHz dBM watts | Amps 2nd and | wans
225 +243 0 16 23 -35 16.0
250 +244 0 16 23 -35 200
275 +243 0 16 28 -35 18.0
Elil] +24.0 0 16 -30 -29 16.0
5 +239 0 16 32 28 157
350 +239 0 16 36 25 188
75 +24.0 0 16 ) 32 16.0
400 +24.7 0 16 43 36 14.0

[Two Tone Test, 2 watts each tane @ 350 & 351MHz 3rd =-38dbe

1P3 = +52




