[image: image4.jpg]Technologies, Ing



[image: image4.jpg]









[image: image5]

[image: image5]Linear RF Amplifier
· Frequency Response: 3- 90 MHz
· Useable Frequency Response 1-100 MHz
· Linear Power: 50 watts 
· Saturated Power: 100 watts 
· Gain: 45 dB
Description:
Designed for linear application in the 1 MHz to 100 MHz range. This amplifier utilizes RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. 
Suggested applications: CW, multi-carrier, pulse, PM, AM & FM modulation.










Updated: 0609
	ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	3
	
	90
	MHz

	Power Output Saturated 
	Psat
	
	100
	
	Watt

	Power Output P-1dB 
	P-1dB
	
	50
	
	Watt

	Gain
	G
	43
	45
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	±1
	
	dB

	Input VSWR
	S11
	
	
	1.3:1
	-

	Harmonics @ 50watts 2nd/3rd 
	H
	
	-40/-24
	-35/-20
	dBc

	Inter-modulation Point

2 Tones, 5W per tone @  50 & 51MHz
	IP3
	
	57
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current @ 50watts
	Amps
	
	7
	
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical 1ms OFF, 10ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	3.0” x 8.0” x 1.25”
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through
	-
	-

	Cooling
	Heat-sink not included
	-
	-

	Weight
	1.25
	Typ
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 70°C with self reset.
	Typ

	Input Overdrive
	Fold-back overdrive protection to 20dBM
	Max

	Load VSWR
	4:1 up to 50 watts
	Max

	Reverse Polarity Protection
	Included
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+50°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curves
[image: image6.jpg]TrLSI1 Log Mag 5.000d6 Ref 0.000d8 [R5]
MIGE S21 153 Maj 1.000d6/ Rar 43.0048 [RT]

T.G000000 iz
30000000 iz
50.000000 itz
T0i00000 iz

T E
priceiRd
35705 o
3700

1 Start 500 kHe

oW 70 ke

Stop 110 MH:z I




Small Signal Frequency Response Curve
[image: image1.jpg]Tr1SI1 Log Mag 5.000d6 Ref 0.000d8 [R5]
MI S21 153 Naj 1.000d6/ Rar 43.0048 LRT]
45,00
T 10000000 iz 43001 &
2 30000000 Mz 34247 &
3 30.000000 iz 3422 o
47.00 |53 10300000 Wiz —43:656 o8
46.00
45.00
44,00 3
i3 3
43.00 "
42,00
4100
40,00
.00
A
.00 s
1 Start 500 kHe TFBW 70 kHz Stop 110 MH:z I





Frequency Response Curve @ 50 Watt Output
Response Curves
	[image: image7.jpg]




	Frequency Response Curve @ 100 Watt Output 

	[image: image2.png]MKR A —980@ kHz
-40 .05 dB

A0 40: 31 JUN 23, 2gg9
P

o

REF —-4.¢ dBm

PEAK

L.OG

!
1

i

1

[

|

i

b
TN
Al
P

SPAN 6.000 MHz

A

! —
i

BN
%
¢
|

i

T T T T e
|
|

LA
iy

H
i
i
i
i
{
i
§
H
i
i
i
!
\
.

H
i
: i i
§ SUU SISO SR
tﬁf;
!

b

}

“

ATTEN 1@ dB

T
-
B
i ]
i
!
!

i —

| MARKER A

| -890 kHz

—40.05 dB
|

|
|
|
b
{
}
|
|

CENTER 5¢.500 MHz

{
WA SB| |
sC FCL.

10
dB/
CORR

SWP 20.¢ msec

VBW 30 kHz

RES BW 30 kHz





	             Two Tones 5 Watts Avg.  Per Tone @ 50 & 51Mhz IP3 = 57dBm




Response Curves

[image: image8.jpg]TrLSI1 Log Mag 5.000d6 Ref 000038 [R5]
MIGE S21 153 Maj 1.000d6/ Rar 43.0048 [RT]
45,00
T 10000000 iz 90957 4
2 30000000 Mz 32.034 o8
3 30.000000 Mz 32230 o
47.00 |53 03 00000 Wiz 41600 o
46.00
45.00
44,00
43.00)|
A
42,00 5
i
4100
40,00
.00
A
.00 s
(1 Stan 500 kHz TFBW 70 kHe Stop 110 MHz IEL!





[image: image9.emf]HARMONICS @ 50 Watts

-55

-50

-45

-40

-35

-30

-25

-20

0 10 20 30 40 50 60 70 80 90 100 110

Freq. (MHz)

(dBc)

2nd 3rd

   



Outline Drawing
[image: image3.png]0.81" [2.06¢cm]

*—_‘1_ 1.28" [3.24cm]

3.00" [7.62cm]

20.157" [30.40cm]

+28VDC
RF Output |[[ ]

www.nptrf.com

Model # NP-520
Serial # xxxx

8.00" [20.32¢cm] 7.726" [19.62cm]

NP Technologies, Inc.

Shut Off

= |

0.137" [0.35¢cm] —t

0.137" [0.35cm]
2.726" [6.92cm]

0.79" [2.01cm] ~{ l<— 0.46" [1.17cm]

T ———

1.25" [3.17cm]













2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image10.emf]COMPRESSION @ 50 Watts

0.40

0.50

0.60

0.70

0.80

0.90

1.00

1.10

1.20

1.30

1.40

0 10 20 30 40 50 60 70 80 90 100 110

Freq.(MHz)

(dB)

