[image: image2.jpg]Technologies, Ing



[image: image2.jpg]









[image: image3]

[image: image3]Linear RF Amplifier
· Linear Power: 300 watts (700-960MHz band)

· Usable Frequency Response: 500-1000 MHz
· Saturated Power: 400 watts (700-960MHz band)
· Gain: 55 dB
Description:
 Designed for linear application in the 700 to 960 MHz range. This amplifier utilizes RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. 
Suggested applications: CW, multi-carrier, pulse, PM, AM & FM modulation.











Updated: 0709
	ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	700
	
	960
	MHz

	Power Output Saturated (700-960MHz)
	Psat
	
	400
	
	Watt

	Power Output P-1dB (700-960MHz)
	P-1dB
	
	300
	
	Watt

	Gain
	G
	53
	55
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	±0.5
	±1.2
	dB

	Input VSWR
	S11
	
	
	1.4:1
	-

	Harmonics @ 300watts
	H
	
	-50
	
	dBc

	Inter-modulation Point

2 Tones, 25W per tone @  900 & x901MHz
	IP3
	
	+64
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current @ 300watts
	Amps
	
	47
	53
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical 1ms OFF, 10ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	12 x 10.75 x 1.3
	Max
	Inch

	RF Connectors IN/OUT
	SMA in, N out
	-
	-

	DC Connectors
	High Power D-sub / with mate
	-
	-

	Cooling
	Heat-sink not included
	-
	-

	Weight
	8.5
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 70°C with self reset.
	Typ

	Input Overdrive
	Fold-back overdrive protection to 20 dBm
	Max

	Load VSWR
	4.0:1 up to 300 watts
	Max

	Reverse Polarity Protection
	None
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+50°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curves
[image: image4.jpg].00

Tr1SI1 Log Mag 5.000d6 Ref 000038 [F1]
MU S21 153 Mag 1.000d6/ Rer 530048 LT

s7.00
.00
ss.00
s6.00
s2.00)
s2.00
.00
.00

49.00

4500

T 50000000 Wz 5.5 9
-2 1.0000000 Gz 55635 o
2
]
g
=
TFBW 70 kHz Stop 1.05 GHz S|

(1 Start 450 MHz





Small Signal Frequency Response Curve
Broadband  500-1000MHz
[image: image5.jpg]



Small Signal Frequency Response Curve
700- 960MHz

Response Curves
[image: image6.jpg](1 Start 630 MHz TFBW 70 kHz

TrLSI1 Log Mag 5.000d6 Ref 0.000d8 [R5]
MU S21 153 Naj 1.000d6/ Rar 54.0048 [RT]
s5.00
T 700.00000 iz 5557 4
-2 56000000 Mz 55236 o
se.00
s7.00
.00
2
3
sso0 [T
s4.00)
s.00
s2.00
.00
s0.00
2
as.00 = i

Stop 970 MH:z [





Frequency Response Curve @ 300 Watts Output
Broadband 500-1000MHz
[image: image7.jpg].00

Tr1SI1 Log Mag 5.000d6 Ref 0.000d8 [F1]
MU S21 153 Mag 1.000d6/ Rer 530048 LT

T

s7.00
.00
ss.00
s6.00
s2.00))
s2.00
.00
.00

49.00

4500

50700000 iz
-2 10000000 GHx

ENEET
EXEE

(1 Start 450 MHz

1FBW 70 kHe

=
Stop 1.05 GHz S|





Frequency Response Curve @ 300 Watts Output

700-960MHz

Response Curves

[image: image8.jpg]TrLSI1 Log Mag 5.000d6 Raf 0.000d8 [R5]
MU S21 153 Naj 1.000d6/ Rar 54.0048 [RT]

s5.00

T

se.00
s7.00
.00
ss.00
s4.00)
s.00
s2.00
.00

s0.00

70000000 Wiz
2 56000000 Mz

T
e

T T

2

49.00

zS

T St 630 M

e 70 ke

Stop 970 MHz 1|





Frequency Response Curve @ 400 Watts Output

700-960MHz

[image: image9.jpg]TrLSI1 Log Mag 5.000d6 Raf 0.000d8 [R5]
MU S21 153 Naj 1.000d6/ Rar 54.0048 [RT]

s5.00

T

se.00

s7.00

.00

ss.00

s4.00 >M4

s.00

s2.00

.00

s0.00

49.00

70000000 Wiz
2 56000000 Mz

SEE &
SERE

2

zS

T s e M

oW 0k

Stop 970 MHz I





Two 25 watt CW tones @ 900 & 901 MHz
IP3 = +64dBM
Outline Drawing
[image: image1.png]10.750"

[27.305cm]
— ° ° S ® ° ° I
o . 20.160" \kh_
[#0.406cm]
Mounting Holes
_H_”h RF Input 7 Places
o o o
° o o o
° ° NP Technologies, Inc. ©
www.nptrf.com
Model # NP 525
Serial # xxxx
- o ° o -
£ I £
S 6.075" RF Output //g 5|2
RIR = —® o ° S
=lo [15.430cml] o 3
y ®
- ° ° o o
° o - °
£
sls
o o m u.._ o o
95
° ° b ° ©
GND o
GND [}
Shut down [}
Optional +28 o
_H_”_o<w<m Det. ° +28, [}
o
4 ©
1B ¢ . . . X . . 0@ 1
T
2.825" 7.625"
[7.175cm] 19.368cm]
1298
£3.296cm]
© _ 050"
E [1.397cm /

“














2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image10.png]MKR A 2.025 MHz

V74

—-40 .15 dB

ATTEN 410 dB

REF —B8.4 dBm

r

r@i 3fwi;i-?.i%;L;,,

iiiqzizﬂeii%.ézJi;fAIi:Jil;J;!e

MARKER A
2.¢925 MHz
40 .15 dB

SPAN 6.000 MHz

R 90¢.50@ MHz
#RES BW 10 kHz

o

CEN

SWP 1B8¢ msec

VBW 10 kHz



