[image: image4.jpg]Technologies, Ing



[image: image4.jpg]









[image: image5]

[image: image5]40 Watt Linear RF Amplifier              
· Frequency Response: 20-520 MHz
· Usable Frequency Response 15-530 MHz
· Linear Power: 40 watts
· Sat. Power: 60 watts
· Gain: 45 dB
Description:
 Designed for linear applications in the 20 to  520 MHz range. This instrumentation amplifier utilizes class “A” and “A/B” RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. Suggested applications: CW, multi-carrier, pulse, PM, AM & FM modulation.
	ELECTRICAL SPECIFICATION @ VDD= +28VDC: Temp.=25°C, 50Ω System                                            0309

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	20
	
	520
	MHz

	Power Output Saturated
	Psat
	60
	
	
	Watt

	Power Output P-1dB
	P-1dB
	
	40
	
	Watt

	Gain
	G
	43
	45
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	±1
	±1.3
	dB

	Input VSWR
	S11
	
	1.3:1
	1.6:1
	-

	Harmonics @ 40 Watts, 2nd/3rd
	H
	
	
	-20/-30
	dBc

	Inter-modulation Point

2 Tones, 2W per tone @ 400 & 401 MHz
	IP3
	
	+54
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	24
	28
	30
	Volt

	Operating Current @ 40 Watts / 60 Watts
	Amps
	
	7.5/8.6
	10
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical: 1ms OFF, 10ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	5.30 x 2.50 x 1.05
	Max
	Inch

	RF Connectors IN/OUT
	SMA
	-
	-

	DC Connectors
	Filtered feed-through.
	-
	-

	Cooling
	4” Fan and Heat-sink.
	-
	-

	Weight
	8.5
	Max
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 70°C with self reset.
	Typ

	Input Overdrive
	Fold-back overdrive protection to 20 dBm.
	Max

	Load VSWR
	Infinite up to 20 watts.
	Max

	Reverse Polarity Protection
	Included
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	50°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
[image: image6.emf]COMPRESSION @ 40 Watts

0.00

0.20

0.40

0.60

0.80

1.00

1.20

20 70 120 170 220 270 320 370 420 470 520

Freq.(MHz)

(dB)


Small Signal Frequency Response Curve 
[image: image1.jpg]TrLSI1 Log Mag 5.000d8/ Raf 000038 [R5]
MIGE S21 153 Naj 1.000d6/ Rar 4.0048 [RT]

s0.00

T 75500000 iz 49937 9
»2 52000000 iz 34462 B

49.00
45,00
47,00
46.00
4s.00)|
44.00
43.00
szo0 |{§

400

40.00

(1 Start 300 kHe TFBW 70 kHz

=
Stop 550 MH:z [





Frequency Response Curve @ 40 Watts Output
Response Curves
[image: image2.jpg]TrLSI1 Log Mag 5.000d8/ Ref 000038 [R5]
MU S21 153 Naj 1.000d6/ Rar 43.0048 [RT]

45,00

T 15500000 iz 93349 9
-2 52000000 Wiz 420422 B

47,00
46.00
45.00
44,00
43.00)|
42,00
4100
000 ||

.00

.00

1 Start 300 kHe TFBW 70 kHz

=
Stop 550 MH:z I





Frequency Response Curve @ 60 Watts Output

[image: image3.jpg]MKR A 1.885 MHz

ATTEN 10 dB

.@ dBm

REF

~-43.26 dB

e
i
%
!
e

o
i

= -
{

i

]

|

|
uw,_ﬁ__,;_mmm_,lf
|

—
z

H

i

1

~

|

%

i

B

|

%
|
-

SPAN 6.00d MHz

e
i
|
|
ol
%
|

W
s
M
|
——
M
5
=
|
!
w

YWWWG

iu,\w LS. T S =
SR c.i.,zxc%.i.;!.“.w PRI S

w,—raw-“m
1
|
i
H
i

e e e A A 3 £

e

ey
|
—t

H
§

e
i

o i

i
!
§
{
- i o i, A A e
T
SUSHEUIS SN W,Wm A~

TR
2
|
|

ISR A

S R

T
|
| MARKER A
| 1.995 MHz
| ~43.26 dB
T
|
S—

| |
i ¢
A
% H

|

R
i i

{

{

i

]

e St

|

!

i

H

—
|
|
i—-—-« SESRP—
|
|
o
|
i
|
|
H
P
%
D
WA SB|
FC|
CORR|

rd

AK
CENTER 400.500 MHz

PE
L.OG
10
dB.,
SC

SWP 6.0 msec

#VBW 10 kHz

#RES BW 30 kHz




Two Tones 2 Watts Avg.  Per Tone @ 400 & 401MHz

IP3 = +54dBm

Response Curves

[image: image7.jpg]


[image: image8.jpg].00

TrLSI1 Log Mag 5.000d6 Ref 000038 [R5]
MU S21 153 Maj 1.000d6/ Rar 45.0048 [RT]

49.00
45,00
47,00
46.00
4s.00)|
44,00
43.00
42,00

100

40.00

1 Start 300 kHe

T 15500000 iz 96487 4
-2 52300000 Wiz 35750 B
2
2
=
TFBW 70 kHz Stop 550 MH:z 5






Outline Drawing
[image: image9.jpg]THRU 4 PLACES.

Ground /ShutDFF 5025 +28vdc. / Graund
\ o o L \ o
A |
2,500 @ Eiee0:
‘ L

Ofren By G0

0486 | o M, o

1.000
L]

LI
RF INPUT 1‘0.50 o

5.300
6.170

B
Ty RF OUTPUT




[image: image10.emf]HARMONICS @ 40 Watts

-60.00

-50.00

-40.00

-30.00

-20.00

-10.00

0.00

20 120 220 320 420 520

Freq. (MHz)

(dBc)

2nd 3rd


Also available on Heat-Sink.









2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

All specifications subject to change without notice

[image: image11.jpg]


